Lucas Ranch Homeowners Association, Inc.

THIS FENCING ADDENDUM REPLACES THE LANGUAGE INCLUDED IN THE ARCHITECTURAL & DESIGN GUIDELINES

H. Fencing and Walls

The materials, height, location and construction of all fences must be approved in advance by the Reviewer.

- Lot Fencing. All lot fencing shall be submitted for approval by the ACC.

- Retaining Walls. All retaining wall plans over four feet (4’) tall or within any right of-way must be submitted to the Reviewer for consideration and must be constructed in accordance with stamped engineered plans by a licensed engineer.

- Plans. Plans submitted for fences or walls must be drawn on an accurate copy of the site plan.

- Greenbelt and Common Area / Pond Fencing. Unless otherwise approved in writing at the sole discretion of the Reviewer, all lots backing up to a greenbelt or Common Area / Pond must be constructed to not obstruct other residents’ views of the common areas and in such a way to maintain the natural feel of the subdevelopment.. Requirements for fencing bordering these areas include:

  - Maximum height of 4’.

  - Privacy fencing is prohibited; approved fencing style includes x4 board post fence, picket fence, and others as approved by the ACC.

  - Must be setback from bordering lot line by 20’

- Construction. All privacy fencing between Lots must be Good Neighbor-style wood fencing. Good Neighbor fencing is defined as wood fencing that alternates the side the panel faces every eight feet (8’). Fences shall utilize metal posts or a minimum of 4”x 4” posts and must have at least three horizontal boards for attaching the pickets. Fences that side onto collector roads or otherwise identified high visibility locations will require the fence to be capped.

- Setbacks. The side yard fence at the point where it connects to the house must be setback a minimum of fifteen feet (15’) from the front corner of the house and a minimum of ten feet (10’) from the back corner of the house. All side yard fences must be installed so that they are perpendicular to the house unless circumstances on site dictate otherwise. Side yard fences must be installed so that all pickets are facing toward the street and no fence rails are seen from the street.

- Stain. All side-yard fences may only be stained using a stain that is approved in advance by the Reviewer. Any part of the fence that is visible from any street shall be routinely re-stained no less than every four years) in the approved stain color and the Reviewer and/or the Association shall have the right to re-stain such visible portion of the fence and charge the expense to the Owner pursuant to the terms and provisions of the Declaration.

- Prohibited Elements:

- Shadowbox or “Pallet” type fencing

- Fencing of front yards.

- Solid walls enclosing an entire site.

Page 2 of 2